|  |  | ### 67D SERIES |  |  | ## WIDE INPUT VOLTAGE RANGE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ## 15 WATTS DC-DC CONVERTER |  |  |  |  |  |  |
|  | FEATURES |  |  |  |  |  |  |  |  |  |  |
|  |  2:1 WIDE INPUT VOLTAGES:9-18V,18-36V,36-72V  15 WATTS (2x1x0.4inches) PACKAGE  EFFICIENCY TO 80%  P1 INPUT FILTER |  |  |  |  |  |  |  |  |  |  |
|  | # GENERAL SPECIFICATIONS |  |  |  |  | ELECTRICAL SPECIFICATIONS |  |  |  |  |  |
|  |  SWITCHING FREQUENCY |  |  | : 300KHz TYP. |  |  INPUT VOLTAGE RANGE |  |  | :12V nominal …9-18V |  |  |
|  |  OUTPUT RIPPPLE & NOISE |  |  | : 100mVp-p(20MHz B.W.) |  |  |  |  | :24V nominal …18-36V |  |  |
|  |  ISOLATION VOLTAGE |  |  | : 1500VDC |  |  |  |  | :48V nominal …36-72V |  |  |
|  |  ISOLATION RESISTANCE |  |  | :109 ohms |  |  OUTPUT VOLTAGE ACCURACY |  |  | :±2% |  |  |
|  |  STORAGE TEMPERATURE RANGE |  |  | :-40℃ TO 100℃ |  |  LINE REGULATION |  |  | :±1.0%(HL-LL) |  |  |
|  |  OPERATING TEMPERATURE RANGE |  |  | :-25℃ TO +71℃ |  |  LOAD REGULATION(SINGLE) |  |  | :±1.0%(100% to 10%) |  |  |
|  |  SHORT CIRCUIT PROTECTION |  |  | : CONTINUOUS |  |  LOAD REGULATION(DUAL) |  |  | :±2%(100% to 10%) |  |  |
|  |  CASE MATERIAL |  |  | : BLACK COATED COPPER WITH |  |  EFFICIENCY |  |  | :see table |  |  |
|  |  |  |  | NON-CONDUCTIVE BASE |  |  EMI/RFI |  |  | :six-sided continuous shield |  |  |
|  | PART N0. |  | INPUT RANGE |  | OUTPUT VOLTAGE |  | OUTPUT CURRENT |  |  | EFFICIENCY (%) |  |
|  | 67D-12S05R |  | 9-18VDC |  | 5VDC |  | 3000mA |  |  | 81 |  |
|  | 67D-12S12R |  | 9-18VDC |  | 12VDC |  | 1250mA |  |  | 83 |  |
|  | 67D-12S15R |  | 9-18VDC |  | 15VDC |  | 1000mA |  |  | 83 |  |
|  | 67D-12D12R |  | 9-18VDC |  | ±12VDC |  | ±625mA |  |  | 83 |  |
|  | 67D-12D15R |  | 9-18VDC |  | ±15VDC |  | ±500mA |  |  | 83 |  |
|  | 67D-24S05R |  | 18-36VDC |  | 5VDC |  | 3000mA |  |  | 82 |  |
|  | 67D-24S12R |  | 18-36VDC |  | 12VDC |  | 1250mA |  |  | 84 |  |
|  | 67D-24S15R |  | 18-36VDC |  | 15VDC |  | 1000mA |  |  | 84 |  |
|  | 67D-24D12R |  | 18-36VDC |  | ±12VDC |  | ±625mA |  |  | 84 |  |
|  | 67D-24D15R |  | 18-36VDC |  | ±15VDC |  | ±500mA |  |  | 84 |  |
|  | 67D-48S05R |  | 36-72VDC |  | 5VDC |  | 3000mA |  |  | 83 |  |
|  | 67D-48S12R |  | 36-72VDC |  | 12VDC |  | 1250mA |  |  | 84 |  |
|  | 67D-48S15R |  | 36-72VDC |  | 15VDC |  | 1000mA |  |  | 84 |  |
|  | 67D-48D12R |  | 36-72VDC |  | ±12VDC |  | ±625mA |  |  | 84 |  |
|  | 67D-48D15R |  | 36-72VDC |  | ±15VDC |  | ±500mA |  |  | 84 |  |
|  | ###### MARKINGS AND DIMENSIONS |  |  |  |  |  | ###### PIN CONNECTION |  |  |  |  |
|  |  |  |  |  |  |  | PIN# | SINGLE |  |  | DUAL |
|  |  |  |  |  |  |  | 1 | +VIN |  |  | +VIN |
|  |  |  |  |  |  |  | 2 | -VIN |  |  | -VIN |
|  |  |  |  |  |  |  | 3 | +VOUT |  |  | +VOUT |
|  |  |  |  |  |  |  | 4 | No Pin |  |  | Common |
|  |  |  |  |  |  |  | 5 | -VOUT |  |  | -VOUT |

Dimensions:inches/mm

Unless otherwise specified, all tolerances are ±.005/±0.13